- Type C connector對地短路事件

| Case 1 |  |
| --- | --- |
|  | Artemis Jira-538 |
| Situation | UART Bridges missed in BmcUSBCountCheck of FFT during PVT build. FR is 0.46% in MP stage |
| Task | Solving UART bridge missing Issue since PVT stage |
| Action | - Check “select pin”, found that it was short to GND without USB plug in. - Cut connection to find the shorting point. - Seem’s the “short” is on the Type C connector. - Sink current into this pin can fix the short. - Re-mount also can fix it. -> looked like mounting issue. - Do X-ray and board slice to check, but can’t find the shorting path. - Analyze manufacture build info, the issue only occurred on “Lotus”. - Conduct a full inspection of the "Lotus" connector before the PCBA build. The defects were found in the material before the SMT process. - RMA the defected connector. And ask for improvement plan. |
| Result | The issue was caused by a defective connector. The vendor helped fix it and then allowed it to be returned for production. |

- PDB Log 誤報事件

| Case 2 |  |
| --- | --- |
|  | Artemis Jira-183 |
| Situation | After sled cycle, that have chance will record "Record time delay 300ms, FRU: 1 , enable HPDB_HS1_BUSBAR_EN" in bmc sel |
| Task | Investigate the root cause of issue log |
| Action | - Replicate the issue overnight, can be reproduce easily. - Measure on the issue net, but scope can’t capture it when issue occurred. - The issue FR is positive relation to the BMC pulling frequency. ->Should be related to I2C bus. - Check I2C signal, found that when issue occurred, the “SDA” seems to be “Change to CLOCK”. - Check i2c path, found that the “SDA” net short to “SCL” of others I2C BUS. |
| Result | The issue cause by the defected 4C+ connector. And it’s single case. |

“附圖展示當初找到short的位置”

- BUSBAR事件

| Case 3 |  |
| --- | --- |
|  | Artemis Jira-640 |
| Situation | [MP] System intermittently encountered unexpected reboot or HPDB power fault during L10 testing |
| Task | Find the root cause of the unexpected reboot |
| Action | - The issue seemed random in the early stages and did not follow a specific PCBA, L10, or rack. - No conclusion was reached after extensive swap testing. - The factory site reviewed security camera footage and found that the issue always occurred when an operator inserted or removed another L10 in the same rack. - Suspected a power pin mating issue between the L10 and the rack BUSBAR. - On-site signal measurements revealed voltage jitter on the "sense pin" when inserting the bottom L10 while measuring on the top L10. |
| Result | Since the Artemis L10 is heavier than other projects, and there were only three L10 units in one rack, the vibration during insertion or removal of the L10 has a greater impact than in other projects. Therefore, it is necessary to lower the sensitivity of the BUSBAR monitor sense pin. (Increase the filter capacitance) |

“推最底層L10進入時，上層L10抓到的signal drop”